Products available for this survey area

- Sidescan sonar imagery (GeoTIFF)

- Bathymetric point data from Singlebeam Echosounder (ArcGIS Shapefile and CSV)

- SV data from Singlebeam Echosounder (ArcGIS Shapefile and CSV)

- Classified thematic habitat map of submerged habitats (ArcGIS shapefile)

- Interpolated DEM from bathymetric point data (Raster)

  - Data has some noise/streaking, so use in automated analyses is not suggested

- Peer reviewed journal publication

  - Legare, B. and C. Mace. 2016. Mapping and classifying eastern oyster habitat in Copano Bay, Texas, by coupling acoustic technologies. Journal of Coastal Research.

Field Data Collection

- Data were collected from 4 September 2013 to 25 February 2014

- Sidescan = Teledyne Benthos C3D

  - Bow-mounted

  - 200 kHz frequency

  - Range of 100 meters

  - 12% Overlap between transect

  - Transect spacing of 185 meters

  - Data collected in WGS 84

  - Projected to UTM 14N

  - Location Data: Ashtec dGPS receiver with Communication System International MBX-3 Differential

- Singlebeam = Biosonics DTX

  - 120 kHz frequency

  - Collected in Visual Acquisition

  - Beam width = 8.1o

  - Pulse rate = 5

  - Pulse duration = 0.1

  - Power Reduction = -9.2

  - Transducer depth = 0.61 m

  - Location Data = Garmin GA 29 GPS

- Survey planning in Hypack

Data Post Processing

- Sidescan

  - Chesapeake SonarWiz V6

  - Bottom track

  - Empirical Gain Normalization

  - Mosaic and output as 8-bit GeoTiff with 0.5 m-resolution

  - WGS84 UTM 14N

- Singlebeam

  - Processed in EchoView

  - Bottom Line Selection

    - Min SV for pick = -30

    - Backstep @ 0

    - Peak threshold = -40

  - Bottom Classification (to pull features)

    - Distance between intervals = 3 m

    - Background noise = -70

  - Bathy file metadata:

    - “Depth_TrnCor” = raw range data converted to depth by adding the transducer offset of 0.61 m

    - “Dep_TidCor” = “Depth_TrnCor” corrected for tide

      - Corrected to MLLW using 1-hour intervals from nearest tide station = Rockport, 8774770

    - All depths reported in meters

DEM Creation (can be re-created from point data using different interpolation techniques)

- Empirical Bayesian kriging

  - Output cell size 50

  - Logempirical transformation

  - Exponential semivariogram

  - 500 points in each local model

  - Local model overlap 3

  - 50 simulated semivariograms

  - Standard circular search pattern

    - Radius of 100 m

    - Maximum neighbors = 500

    - Minimum neighbors = 100

    - Angle 45

    - Sector Type - 4

Habitat Classification

- Manual interpretation based on sidescan imagery and singlebeam SV values

- Accuracy assessment using poling

- 150 ground-truthing samples

  - User’s accuracy

    - Mud = 100%

    - Oyster = 90%

    - Sand = 63%

    - Shell = 69%

  - Producer’s accuracy

    - Mud = 93%

    - Oyster = 89%

    - Shell = 83%

    - Sand = 78%

  - Overall accuracy = 89.6%