Field Data Collection

- Data were collected from 02 January 2017 through 13 December 2017

  - 39 survey days; 1,558 man-hours

- Sidescan = Teledyne Benthos C3D

  - 200 kHz frequency

  - Range of 50 meters

  - 15% Overlap between transect

  - Transect spacing of 85 meters

  - Data collected in WGS 84

  - Projected to UTM 14N

  - Location Data: Ashtec dGPS receiver with Communication System International MBX-3 Differential

- Singlebeam = Biosonics DTX

  - 120 kHz frequency

  - Collected in Visual Acquisition

  - Pulse rate = 8

  - Pulse duration = 0.1

  - Power Reduction = -9.2

  - Transducer depth = 0.61 m

  - Location Data = Garmin GA 29 GPS

Data Post Processing

- Sidescan

  - Chesapeake SonarWiz V7

  - Bottom track

  - Empirical Gain Normalization

  - Mosaic and output as 8-bit GeoTiff with 0.2 m-resolution

  - WGS84 UTM 14N

- Singlebeam

  - Processed in EchoView

  - Bottom Line Selection

    - Min SV for pick = -9

    - Backstep @ -15 discrimination level

    - Peak threshold = -13

  - Bottom Classification (to pull features)

    - Distance between intervals = 5 m

    - Background noise = -999

    - Bottom echo threshold @ 1 m = ranges from -60 to -40

  - Depth corrected to MLLW in 1-hour intervals from nearest NOAA tide station = Port O’Connor 8773701

DEM Creation (can be re-created from point data using different interpolation techniques)

- Empirical Bayesian kriging

  - Output cell size 50

  - Logempirical transformation

  - Exponential semivariogram

  - 500 points in each local model

  - Local model overlap 3

  - 50 simulated semivariograms

  - Standard circular search pattern

    - Radius of 100 m

    - Maximum neighbors = 500

    - Minimum neighbors = 100

    - Angle 45

    - Sector Type - 4

Thematic Mapping

- Manual interpretation based on sidescan imagery and depth, hardness, and roughness bottom features from singlebeam echosounder

- Accuracy assessment using a combination of petite ponar, dredge, and patent tongs

  - 174 samples

  - Accuracy for Oyster habitats > 80%